## 1) Data preparation and evaluation protocol (constant across experiments)

### Data sources and representation

All experiments start from preprocessed *_combined.npy files containing EMG, force components (Fx, Fy), nominal/MCP angle labels, and segment metadata. Main directories used were:

- data/preprocessing/P3_combined (intramuscular EMG, iEMG)

- data/preprocessing/P3_combined_surfaceEMG (surface EMG, sEMG)

- data/preprocessing/fake_data_extreme (synthetic validation data)

For each bin size, features are extracted (primarily rms_matrix, 16-D) using a consistent preprocessing pipeline (binning and RMS windowing). The dataset loader returns:

- X_by_mode[MODE] (feature matrix)

- y targets (typically [Fx, Fy, Angle])

- groups (plateau/trial group IDs)

- nominal_angles (label per sample).

### Leakage-controlled holdout (the key constant)

All results use the same group-aware holdout:

- For each nominal angle bucket, one plateau group is held out for testing.

- Training uses the remaining plateau groups.

This ensures train/test do not contain near-duplicate bins from the same plateau segment.

### Metric

Performance is reported as circular MAE (degrees) to correctly handle wrap-around (e.g., 359° vs 1°).

## 2) Experiments comparing different targets

### Targets evaluated

We compared two ways to predict angle:

A) Angle-only regression

- Model predicts angle directly (1D).

B) 3-target regression [Fx, Fy, Angle]
Two angle readouts:

- Direct angle head: take the model’s predicted angle output.

- Vector-derived angle: compute angle from predicted force components:
[
\hat{\theta} = \mathrm{atan2}(\hat{F_y}, \hat{F_x})
]
This encourages physically consistent geometry and often helps.

## 2.1) First results (initial “baseline” findings)

### iEMG (P3), rms_matrix, bins ≈ [10ms, 50ms, 100ms]

Across bin sizes [102, 512, 1024]:

- Angle-only ridge: mean MAE 22.29°

- 3-target ridge (vector-derived angle): mean MAE 20.60° (best of the ridge variants)

- 3-target ridge (direct angle output): mean MAE 22.29° (≈ no gain vs angle-only).

Bin size mattered strongly (vector-derived):

- 10ms: ~26.79°

- 50ms: ~18.96°

- 100ms: ~16.04°
→ longer bins improved stability/averaging.

Angle buckets behaved differently:

- 135° comparatively easy (~7.9° mean in these summaries)

- 90° hardest (~32.4°)

- 180° improved substantially with vector-derived angle.

### sEMG baseline (angle-only)

On surface EMG, angle-only performance was worse overall:

- Mean MAE across bins [512, 1024, 2048, 3072]: 32.04°

- Longer bins helped but remained noisier than iEMG.

Takeaway: While vector-derived targets and longer bins improved results, the overall error levels were still not satisfactory—especially given that the task is only three nominal angles.

## 3) Motivation: Results not satisfactory → sanity-check with fake data

### Why fake data?

Even after target engineering and bin tuning, the remaining error suggested a potential representation/linearity limitation: maybe the RMS feature space does not support a near-perfect linear mapping to angle. To test what is fundamentally achievable, we created a controlled synthetic dataset.

## 3.1) How the fake data was created

Core idea: keep realistic EMG time structure by starting from real recordings, but enforce very distinctive channel-amplitude signatures per nominal angle.

Steps:

- Template selection: use real 90Deg_*_combined.npy files from P3_combined as EMG bases.

- Channel-wise scaling profiles (16-D):

  - 90°: descending profile (early channels high)

  - 180°: ascending profile (late channels high)

  - 135°: mid-channel “Gaussian bump”

- Make it extreme: normalize profile → power transform → map to a large scale range (MIN_SCALE to MAX_SCALE), optionally sparsify to top-k channels.

- Patch metadata/targets for consistency:

  - set meta["nominal_angle"]

  - rotate (Fx, Fy) direction to match nominal angle while preserving magnitude

  - set MCP_Angle

  - shift/align angle_achieved time series (circular mean alignment).

## 3.2) Lower bound for linear predictor on fake data

Even on this synthetic “easier” dataset, the linear ridge angle model did not become perfect:

- Example: fake_data_extreme, bin_len≈1024 (~100ms), angle-only ridge MAE ≈ 11.14°.

Interpretation: the pipeline can learn something reliably, but linear models still hit a floor unless the data is made extremely separable—suggesting a structural limitation rather than “just not enough training.”

## 3.3) Is the fake data linearly separable in feature space?

We ran explicit feature separability diagnostics on the binned features:

- Mean feature vector per class (90/135/180): looked strongly different “by eye,” suggesting that the classes have different centroids.

- Centroid distance vs within-class spread:

  - centroid_dist ≈ 21.06

  - within ≈ 6.83

  - ratio ≈ 3.08
This indicates centroids are separated, but within-class variance remains large, implying overlap and no clean linear boundary.

- Conclusion: Even for exaggerated synthetic data, overlap persists → no guarantee of near-perfect linear prediction.

## 3.4) Linear separability check on real data (PCA on P3_combined)

We then repeated separability checks on the real dataset using PCA on rms_matrix features (bin_len=1024 ≈ 100ms), visualizing four views:

- Raw X colored by nominal angle
→ showed multiple regimes/bands rather than three clean clusters.

- Raw X colored by force magnitude |F|
→ strongest PCA direction aligned with |F| (activation intensity dominates variance).
EVR: [0.8064, 0.1255], cumulative ≈ 0.932.

- Row-L2 normalized X colored by nominal angle
→ removing magnitude reveals stronger angle structure (angle encoded in relative channel patterns), but overlap remains.
EVR drops to [0.3771, 0.2928], cumulative ≈ 0.670.

- Row-L2 normalized X colored by |F|
→ confirms that raw-space structure is heavily magnitude-driven.

Interpretation: In real RMS feature space, angle is entangled with stronger factors (especially magnitude/force), producing overlap and regime structure. A single linear mapping is forced into a compromise → consistent with the observed error floor.

## 4) Moving beyond linear: Kernel methods + small neural network

### Why these choices?

Given the evidence for non-linear / multi-regime structure, we moved to models that can learn:

- non-linear interactions between channels,

- piecewise mappings (different regimes),

- smoother representations of circular outputs.

We also adopted a more robust output representation:

- predict (sinθ, cosθ) and recover θ by atan2 (avoids angle wrap discontinuity).

## 4.1) Architecture choices

### Kernel Ridge Regression (RBF)

- Non-linear baseline with smooth function class.

- Tuned gamma (kernel width) and alpha (regularization) using GroupKFold on TRAIN only, centered around a median-distance heuristic for gamma.

### Small MLP (MLPRegressor)

- Lightweight neural net: ReLU MLP with StandardScaler, early stopping.

- Baseline: (64, 64) hidden units.

- Extension: temporal context window k, created by concatenating k consecutive bins within the same plateau group (predict center bin).
Tuned k and hyperparameters via repeated “holdout-like CV” on TRAIN only.

## 4.2) Results (real iEMG P3, rms_matrix, bin_len=3072 ≈ 300ms)

- Tuned Kernel Ridge (RBF) on achieved_sincos: MAE ≈ 10.34°
(per-bucket example: 90° ~5.13°, 135° ~18.03°, 180° ~7.91°)

- MLP baseline (k=1) on achieved_sincos: MAE ≈ 10.11°

- Best tuned MLP + context window:
Best config: k=7, hidden (64,64), alpha=1e-5, lr=0.003
Holdout MAE ≈ 9.35° (best in notebook)
(90° became very accurate; 135° remained the dominant error source).

Final takeaway: The separability diagnostics explain why ridge saturates: the RMS feature space is not cleanly linearly separable/predictable w.r.t. angle. Introducing non-linear capacity (KRR/MLP) and circular-friendly targets improves performance, with the best result coming from a small MLP with short temporal context.